User Guide of MBI toolbox

Introduction

The MBI toolbox provides morphological building index (MBI) [1,2] and several post-processing algorithms [3], which are used for building extraction from very high resolution remote sensing imagery.

This document describes the functionality and usage of the functions in MBI toolbox.

Overview

The basic idea of MBI is to build a relationship between the implicit characteristics of buildings (e.g., brightness, size, and contrast) and the morphological operators (e.g., top-hat by reconstruction, granulometry, and directionality). The function MBI in MBI toolbox is implemented to calculate the MBI feature of the input image. MBI is effective in automatically detecting buildings from high-resolution imagery without selecting any samples, especially when the spectral, contextual, and geometrical information are simultaneously considered, and can be successfully applied to large high-spatial-resolution images. The function MBI_Spectral , MBI_Shadow, MBI_Shape represent refining the MBI with spectral constraints, shadow verification and shape constraints, respectively. In order to detect buildings from the input image, the function MBI, MBI_Spectral , MBI_Shadow, MBI_Shape can be executed sequentially. However, the function MBI_Spectral and MBI_Shadow can be used only when the input image has 4 bands (band 1:Blue band 2:Green band 3:Red band 4:Near Infrared). Otherwise, initial buildings are extracted by performing a threshold on the MBI feature image,  and then can be refined by the function MBI_Shape. Besides, the step of shadow verification should be left out when there are quite a few buildings without the according shadows in the scene. In addition, the optimal resolution for MBI is between 1.0-4.0 meters. MBI can also be applied to multi-index learning, building change detection, urban change detection, urban villages detection and analysis [4, 5, 6, 7].

Functions

MBI

The function MBI is used to extract the MBI feature from the input image.

Syntax

F_MBI = MBI (image,resolution,scale_min,scale_max)

F_MBI = MBI (image,resolution)

Description

| Parameter | Value |
| --- | --- |
| image | The input image. |
| resolution | The resolution of the input image. |
| scale_min | The minimal size of the buildings in the input image. If the minimal size is 5m, then scale_min=5. Default: 5 |
| scale_max | The maximal size of the buildings in the input image. If the maximal size is 85m, then scale_max=85. Default: 85 |

MBI_Spectral

The function MBI_ Spectral is used to refine the original MBI by spectral constraints.

Syntax

F_MBI_Spectral= MBI_ Spectral (image,resolution,mbi,Tndvi_min,Tndvi_max)

F_MBI_Spectral = MBI_ Spectral (image,resolution,mbi)

Description

| Parameter | Value |
| --- | --- |
| image | The input image: 4 bands (band 1:Blue band 2:Green band 3:Red band 4:Near Infrared). |
| resolution | The resolution of the input image. |
| mbi | The original MBI feature image. |
| Tndvi_min | The minimal threshold of the NDVI Default value: 0.12 Suggested range: [0.1, 0.16] |
| Tndvi_max | The maximal threshold of the NDVI Default value: 0.2 Suggested range: [0.2, 0.3] |

MBI_Shadow

The function MBI_Shadow is used to refine the candidate buildings by shadow verification.

Syntax

buildings = MBI_ Shadow (image, building_init, direction, Tnir, Trgb, Ts, Tarea)

buildings = MBI_ Shadow (image, building_init, direction, Tnir,Trgb, Ts)

buildings = MBI_ Shadow (image, building_init, direction, Tnir,Trgb)

buildings = MBI_ Shadow (image, building_init , direction)

Description

| Parameter | Value |
| --- | --- |
| image | The input image: 4 bands (band 1:Blue band 2:Green band 3:Red band 4:Near Infrared). |
| building_init | The initial building extraction result (binarization image, 1 stands for building, 0 stands for background). |
| direction | The opposite direction of illumination. Range: [0, 360) |
| Tnir | The threshold which is used to detect shadow in the NIR band. Default value: 300 |
| Trgb | The threshold which is used to detect shadow in the RGB bands(mean) Default value: 400 |
| Ts | The size of the structure element used to verify candidate buildings and their shadows Default value: 11 Suggested range: [7, 15] |
| Tarea | The minimum area of the shadow structure Default value:1 Suggested range: [1, 9] |

MBI_Shape

The function MBI_Shape is used to refine the initial buildings by shape constraints.

Syntax

buildings = MBI_Shape (building_tmp,Tarea, Textent,Tratio)

buildings = MBI_Shape (building_tmp,Tarea, Textent)

buildings = MBI_Shape (building_tmp,Tarea)

buildings = MBI_Shape (building_tmp)

Description

| Parameter | Value |
| --- | --- |
| building_tmp | The temporary building extraction result (binarization image, 1 stands for building, 0 stands for background). |
| Tarea | The threshold of building area (pixels ), according to the resolution and building area. Default value: 9 |
| Textent | The ratio of pixels in the region to pixels in the total bounding box. Default value: 0.12 |
| Tratio | The length-width ratio of 'BoundingBox'. Default value: 7 Suggested range: [7,10] |

Example

%% Step 1: Load Images

addpath('../data')

testimage = imread('Hangzhou.tif');

%% Step 2: Calculate MBI

mbi = MBI(testimage,2,5,85);

%% Step 3: Enhance the MBI Using Spectral Constraints

mbi_spec = MBI_Spectral(testimage,2,mbi);

%% Step 4: Binarization

buildings_init = mbi_spec > 0.12;

%%Step 5: Shadow Verification

buildings_sv = MBI_Shadow(testimage,buildings_init,285,230,440,11,1);

%% Step 6: Shape Constraints

buildings_sc = MBI_Shape(buildings_sv,9,0.12,7);

%% Step 7：Save the result

imwrite(buildings_sc,'testimage_buildings.tif','tif');

References

[1] Xin Huang, and Liangpei Zhang. A multidirectional and multiscale morphological index for automatic building extraction from mutispectral GeoEye-1 imagery. Photogrammetric Engineering and Remote Sensing, 2011, 77(7): 721–732.

[2] Xin Huang, and Liangpei Zhang. Morphological building/shadow index for building extraction from high-resolution imagery over urban areas. IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, 2012, 5(1): 161-172.

[3] Xin Huang, and Wenliang Yuan. A new building extraction postprocessing framework for high-spatial-resolution remote-sensing imagery. IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, 2016, DOI: 10.1109/JSTARS.2016.2587324.

[4] Xin Huang, Qikai Lu, Liangpei Zhang. A multi-index learning approach for classification of high-resolution remotely sensed images over urban areas. ISPRS Journal of Photogrammetry and Remote Sensing, 2014, 90: 36-48.

[5] Huang Xin, Tingting Zhu, Liangpei Zhang, and Yuqi Tang. A novel building change index for automatic building change detection from high-resolution remote sensing imagery. Remote Sensing Letters, 2014, 5(8): 713-722.

[6] Dawei Wen, Xin Huang, Liangpei Zhang, and Jon Atli Benediktsson. A novel automatic change detection method for urban high-resolution remotely sensed imagery d on multiindex scene representation. IEEE Transactions on Geoscience and Remote Sensing, 2016, 54(1): 609-625.

[7] Xin Huang, Hui Liu, and Liangpei Zhang. Spatiotemporal detection and analysis of urban villages in mega city regions of China using high-resolution remotely sensed imagery. IEEE Transactions on Geoscience and Remote Sensing, 2015, 53(7): 3639-3657.